#Core Kannel Configuration Section

group = core

admin-port = 13000

admin-password = 301164

status-password = 301164

#admin-deny-ip = "*.*.*.*"

#admin-allow-ip = "*.*.*.*"

wapbox-port = 13003

smsbox-port = 13002

log-file = "/var/log/kannel/bearerbox.log"

#box-deny-ip = "*.*.*.*"

#box-allow-ip = "127.0.0.1"

access-log = "/var/log/bearerbox-access.log"

#Wapbox

group = wapbox

bearerbox-host = localhost

log-file = "/var/log/kannel/wapbox.log"

#Smsbox

group = smsbox

bearerbox-host = localhost

sendsms-port = 13013

sendsms-url = "/cgi-bin/sendsms"

sendsms-chars = "0123456789+-"

global-sender = +22177xxxxxxx

log-level = 0

log-file = "/var/log/kannel/smsbox.log"

access-log = "/var/log/kannel/access.log"

#Smsc

group = smsc

smsc = at

smsc-id = Huawei-E1552

#allowed-smsc-id = Huawei-E1552

#port = 13013

modemtype = huawei_e1552

device = /dev/ttyUSB0

pin = 0000

host = localhost

sms-center = +221776380010

my-number = +22177xxxxxxx

#connect-allow-ip = "127.0.0.1"

sim-buffering = true

keepalive = 10

#reroute-dlr = false

speed = 9600

#Modems

group = modems

id = huawei_e1552

name = "huawei E1552"

detect-string = "Huawei"

init-string ="AT+CNMI=2,1,2,2,0"

message-storage = "sm"

speed = 9600