2.2 Stand-alone JTAG Probe

The ZY1000 from Ultimate Solutions is technically not a dongle but a stand-alone JTAG probe that unlike most dongles doesn’t require any drivers running on the developers host computer.  Once installed on a network using DHCP or a static IP assignment, users can access the ZY1000 probe locally or remotely from any host with access to the IP addressed assigned to the probe.  The ZY1000 provides an intuitive web interface with direct access to the OpenOCD debugger.  Users may also run a GDBSERVER directly on the ZY1000 to take full advantage of GCC & GDB to debug any distribution of embedded Linux or netBSD running on the target.  The ZY1000 supports  RTCK & RCLK or adaptive clocking and has a built-in relay to power cycle the target remotely.  For more information, visit:

http://www.ultsol.com/index.php/component/content/article/8/33-zylin-zy1000-jtag-probe

[END]